NPC UNIT 4 ASSIGNMENT REPORT

Configure DHCP and Set Up a Network Using Cisco Packet Tracer

Name: Nandini Samir Nikumbh

PRN – 2125UCSF1081
Subject: Network Protocol Configuration (NPC)

1. Aim

To configure a DHCP server and set up a small network using Cisco Packet Tracer. The main aim is to assign IP addresses automatically to PCs and test network connectivity.

2. Software and Hardware Requirements

- Cisco Packet Tracer software

- One 2960 Switch

- One Server

- Two PCs (PC0 and PC1)

- Copper Straight-Through cables

3. Network Topology

In this practical, I created a simple network using one switch, one server, and two PCs. PC0, PC1, and Server0 were connected to the switch using Copper Straight-Through cables.

The switch connects all the devices and allows them to communicate with each other within the local network.

Screenshot 1: Network topology

4. Server IP Configuration

I configured a static IP address on Server0 to provide DHCP services to the PCs.

The server settings were:

- IP Address: 192.168.1.2

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

- DNS Server: 192.168.1.2

Screenshot 2: Server ip conifiguration

5. DHCP Server Configuration

I opened Server0 and selected the Services tab. Then I selected DHCP and turned the DHCP service ON.

I created a DHCP pool named LAN_POOL and configured the following settings:

- Pool Name: LAN_POOL

- Default Gateway: 192.168.1.1

- DNS Server: 192.168.1.2

- Start IP Address: 192.168.1.10

- Subnet Mask: 255.255.255.0

- Maximum Number of Users: 50

After entering the details, I added the pool to the DHCP server.

Screenshot 3: DHCP Service Configuration

6. PC IP Configuration

I opened the Desktop tab on PC0 and PC1 and selected IP Configuration. Then I selected DHCP to obtain IP addresses automatically.

Both PCs were configured to request their IP addresses from the DHCP server.

Screenshot 4: PC0 IP Conifiguration

Screenshot 5: PC1 IP Configuration

7. Connectivity Testing

To check network connectivity, I opened the Command Prompt on PC0 and used the ping command to test communication with PC1.

Command used:

ping 192.168.1.11

The command returned successful replies after I configured the IP addresses for the connectivity test. This confirmed that PC0 and PC1 could communicate with each other using those settings.

Screenshot 6:

8. Result

The network was created successfully in Cisco Packet Tracer. The DHCP service and LAN_POOL were configured, and the PCs were able to obtain an IP address using DHCP. A successful ping test confirmed connectivity between the two PCs after IP configuration.

9. Conclusion

Through this practical, I learned how to create a simple network in Cisco Packet Tracer, configure a DHCP server, assign IP addresses to PCs, and test network connectivity using the ping command.